🚀 Phase-wise Implementation Plan

Think of this like how you’d run a real embedded sprint.

🔷 Phase 1: Foundation (Bring-up)

Goal:

Basic connection establishment working

Scope:

- Wi-Fi init (SoftAP/Station)

- TCP server start

- Accept single client

Deliverables:

- System reaches: WAIT_FOR_CLIENT → CONNECTED

- Basic connect/disconnect detection

Notes:

👉 No heartbeat, no validation yet
👉 Just ensure pipe is working

🔷 Phase 2: State Machine Implementation

Goal:

Make behavior deterministic

Scope:

- Implement all states:

  - INIT

  - WIFI_READY

  - WAIT_FOR_CLIENT

  - CONNECTED

  - CONNECTION_LOST

  - RECONNECTING

Deliverables:

- Valid transitions only

- State exposed via API

Notes:

👉 This is where system becomes predictable & debuggable

🔷 Phase 3: Connection Health (Heartbeat + Timeout)

Goal:

Detect “fake connections” (TCP alive but app dead)

Scope:

- Heartbeat handling

- Timeout counters

- Connection loss detection

Deliverables:

- Automatic transition to CONNECTION_LOST on timeout

Notes:

👉 This is critical for real-world reliability

🔷 Phase 4: Reconnection Logic

Goal:

Self-recovery system

Scope:

- Retry mechanism

- Backoff strategy

- Max retry limit

Deliverables:

- CONNECTION_LOST → RECONNECTING → CONNECTED

Notes:

👉 Avoid aggressive retry loops

🔷 Phase 5: Access Control (Authentication)

Goal:

Secure connection entry

Scope:

- Handshake validation

- Token/password check

Deliverables:

- Reject unauthorized clients

Notes:

👉 Keep lightweight for now

🔷 Phase 6: Data Flow Control Integration

Goal:

Control system behavior based on connection

Scope:

- Enable/disable data reception

- Interface with Router/Buffer Manager

Deliverables:

- Data only flows in CONNECTED state

🔷 Phase 7: Resource & Error Handling

Goal:

Make system robust

Scope:

- Cleanup on disconnect

- Error reporting

- Logging hooks

Deliverables:

- No memory leaks

- Clear error states

🔷 Phase 8: Configurability & Optimization

Goal:

Make system flexible

Scope:

- Configurable:

  - Timeouts

  - Retry count

  - Heartbeat interval

Deliverables:

- Config-driven behavior

🔷 Phase 9: Real-Time Compliance

Goal:

Make it automotive-grade

Scope:

- Non-blocking design

- Event-driven execution

- Timing validation

Deliverables:

- Deterministic execution